Automatic Street Light using LDR Sensor (Complete Notes)

1. Introduction

An Automatic Street Light system is a smart lighting circuit that automatically turns the street light ON or OFF based on surrounding light intensity.

During the day, the light remains OFF, and at night, it turns ON automatically.

2. Objective of the Project

- To keep street lights OFF during daylight

- To turn street lights ON in darkness

- To reduce human dependency

- To save power and enable automation

3. Components Required

- LDR (Light Dependent Resistor)

- 10kΩ Resistor

- NPN Transistor (BC547 / 2N2222)

- LED / Bulb

- 330Ω Resistor

- Power Supply (5V/9V)

- Breadboard and wires

4. LDR Sensor

An LDR is a photo-sensitive resistor whose resistance depends on light intensity.

Bright light → Low resistance

Darkness → High resistance

5. Working Principle

Uses a voltage divider:

Vout = Vin × (R2 / (R1 + R2))

6. Transistor Role

Acts as a switch:

Base voltage present → ON

No base voltage → OFF

7. Working

Day:

LDR low → transistor OFF → LED OFF

Night:

LDR high → transistor ON → LED ON

8. Advantages

- Energy saving

- Automatic operation

- Low cost

9. Applications

- Street lights

- Garden lighting

- Smart homes

10. Conclusion

This project is simple, cost-effective, and useful for smart automation systems.