# ESP32 Complete Notes (Robotics + IoT)

## What is ESP32?

ESP32 is a powerful low-cost microcontroller developed by Espressif Systems. It is widely used in robotics, IoT, home automation, industrial automation, AIoT projects, and wireless communication. It includes built-in Wi‑Fi, Bluetooth, GPIO, ADC, DAC, PWM, and dual-core processing.

## ESP32 Architecture

ESP32 contains a dual-core Xtensa LX6 processor running up to 240 MHz, integrated Wi‑Fi and Bluetooth, SRAM, flash support, ADC, DAC, PWM, UART, SPI, I2C, and touch sensor support.

## Main Features

- Dual-core processor
- Wi‑Fi and Bluetooth
- Multiple GPIO pins
- ADC and DAC support
- PWM motor control
- Touch sensing
- FreeRTOS multitasking

## ESP32 Variants

ESP32-WROOM, ESP32-WROVER, ESP32-S2, ESP32-S3, and ESP32-C3 are commonly used variants with different features such as AI acceleration, low power operation, and camera support.

## GPIO Pins

GPIO pins are used for LEDs, motors, relays, and sensors. Some pins support PWM, ADC, DAC, UART, SPI, and I2C communication.

## ADC and DAC

ADC converts analog signals into digital values. DAC generates analog output voltage. ESP32 ADC resolution is 12-bit.

## PWM

PWM is used for motor speed control, servo control, and LED brightness adjustment.

## Communication Protocols

ESP32 supports UART, SPI, and I2C communication protocols for interfacing with displays, sensors, RFID modules, GPS, GSM, and other devices.

## Power Modes

ESP32 supports Active, Modem Sleep, Light Sleep, and Deep Sleep modes for low-power applications.

## ESP32 in Robotics

ESP32 is widely used in robot cars, robotic arms, line follower robots, surveillance robots, and drones because of its wireless capability and fast processing.

## ESP32-CAM

ESP32-CAM includes an OV2640 camera module for video streaming, face detection, and surveillance systems.

## Programming ESP32

ESP32 can be programmed using Arduino IDE, PlatformIO, ESP-IDF, and MicroPython.

## Arduino IDE Setup

Install Arduino IDE, add the ESP32 board manager URL, and install the ESP32 board package from Board Manager.

## First Program

Basic LED blink program uses pinMode() and digitalWrite() functions.

## Wi‑Fi Programming

ESP32 can connect to Wi‑Fi networks using the WiFi.h library.

## Bluetooth Programming

ESP32 supports Bluetooth Classic and BLE using BluetoothSerial and BLE libraries.

## ESP32 vs Arduino UNO

ESP32 is much faster and more powerful than Arduino UNO with built-in Wi‑Fi, Bluetooth, more memory, and multitasking support.

## Advantages

- Low cost
- Wireless connectivity
- High speed
- Real-time performance
- Suitable for robotics and IoT

## Disadvantages

- Slightly complex for beginners
- 3.3V logic only
- Some pins reserved for boot

## Common Sensors

Ultrasonic sensor, IR sensor, DHT11, MPU6050, RFID, GPS, gas sensors, and temperature sensors are commonly used with ESP32.

## Motor Drivers

L298N, BTS7960, servo motors, and ESCs are commonly interfaced with ESP32 for robotics projects.

## RTOS in ESP32

ESP32 uses FreeRTOS for multitasking and real-time applications.

## Applications

ESP32 is used in smart homes, industrial automation, robotics, drones, AI systems, and cloud-connected IoT devices.

## Conclusion

ESP32 is one of the best microcontrollers for robotics and IoT due to its powerful processor, wireless features, low cost, and wide community support.

## Sample LED Blink Program

void setup() {
  pinMode(2, OUTPUT);
}

void loop() {
  digitalWrite(2, HIGH);
  delay(1000);
  digitalWrite(2, LOW);
  delay(1000);
}

## Sample Wi‑Fi Program

#include <WiFi.h>

const char* ssid = "YOUR_WIFI";
const char* password = "12345678";

void setup() {
  Serial.begin(115200);
  WiFi.begin(ssid, password);

  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }

  Serial.println("Connected");
}

void loop() {
}