# ESP32 Pins Complete Notes

## Introduction

ESP32 has many GPIO pins used for LEDs, sensors, motors, displays, Wi‑Fi, Bluetooth, and communication protocols.

## Types of ESP32 Pins

GPIO Pins
ADC Pins
DAC Pins
PWM Pins
UART Pins
SPI Pins
I2C Pins
Touch Pins
Power Pins

## GPIO Pins

GPIO pins are used for digital input and output operations such as controlling LEDs, relays, motors, and reading buttons.

## Safe GPIO Pins

GPIO2, GPIO4, GPIO5, GPIO12, GPIO13, GPIO14, GPIO15, GPIO18, GPIO19, GPIO21, GPIO22, GPIO23, GPIO25, GPIO26, GPIO27, GPIO32, GPIO33

## Input Only Pins

GPIO34, GPIO35, GPIO36, and GPIO39 work only as input pins.

## ADC Pins

ESP32 has 12-bit ADC support.
ADC1 Pins: GPIO32, 33, 34, 35, 36, 39
ADC2 Pins: GPIO0, 2, 4, 12, 13, 14, 15, 25, 26, 27

## DAC Pins

ESP32 contains 2 DAC pins:
GPIO25 = DAC1
GPIO26 = DAC2

## PWM Pins

PWM is used for motor speed control, servo control, and LED dimming. Almost all GPIO pins support PWM except GPIO34–39.

## UART Pins

UART0:
TX = GPIO1
RX = GPIO3

UART2:
TX = GPIO17
RX = GPIO16

## I2C Pins

Default I2C pins:
SDA = GPIO21
SCL = GPIO22

## SPI Pins

Default VSPI pins:
MOSI = GPIO23
MISO = GPIO19
SCK = GPIO18
CS = GPIO5

## Touch Sensor Pins

Touch pins:
GPIO0, GPIO2, GPIO4, GPIO12, GPIO13, GPIO14, GPIO15, GPIO27, GPIO32, GPIO33

## Boot Pins

GPIO0, GPIO2, GPIO12, and GPIO15 are boot-related pins. Incorrect connections may stop ESP32 from booting.

## Power Pins

VIN = 5V Input
3V3 = 3.3V Output
GND = Ground

## Reserved Pins

GPIO6 to GPIO11 are reserved for flash memory and should not be used.

## Built-in LED

Most ESP32 boards use GPIO2 for the built-in LED.

## Servo Motor Pins

Recommended servo pins:
GPIO13, 14, 18, 19, 21, 22, 23, 25, 26, 27, 32, 33

## Ultrasonic Sensor Connection

HC-SR04 Example:
TRIG = GPIO5
ECHO = GPIO18

## Motor Driver Connection

L298N Example:
IN1 = GPIO25
IN2 = GPIO26
IN3 = GPIO27
IN4 = GPIO14

## OLED Display Connection

OLED I2C Connection:
SDA = GPIO21
SCL = GPIO22

## Relay Module Pins

Recommended relay pins:
GPIO18, GPIO19, GPIO21, GPIO22, GPIO23

## Important Notes

1. GPIO34–39 are input only.
2. GPIO6–11 are reserved.
3. Avoid incorrect usage of boot pins.

## Conclusion

ESP32 supports digital I/O, analog input, analog output, PWM, UART, SPI, I2C, touch sensing, Wi‑Fi, and Bluetooth, making it ideal for robotics and IoT.

## Example Code: LED Blink

void setup() {
  pinMode(2, OUTPUT);
}

void loop() {
  digitalWrite(2, HIGH);
  delay(1000);
  digitalWrite(2, LOW);
  delay(1000);
}